Link to specs.

Use only ADC1 pins (32-39) for analog inputs, as ADC2 conflicts with Wi-Fi !

| GPIO |  | Input | Output | Safe? | Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | Yes |  | Yes | ⚠️ | Boot strapping pin; LOW at boot enters flashing mode. |
| 1 | Yes |  | Yes | ⚠️ | UART TX (serial programming/logs) |
| 2 | Yes |  | Yes | ⚠️ | Strapping pin; must be sane during boot |
| 3 | Yes |  | Yes | ⚠️ | UART RX |
| 4 | Yes |  | Yes | ⚠️ | Strapping pin; external pulls can break boot |
| 5 | Yes |  | Yes | ⚠️ | Strapping pin; often used as SPI CS safely |
| 6–11 | Yes |  | Yes | ❌ | Connected to SPI flash |
| 12 | Yes |  | Yes | ⚠️ | Flash voltage strapping pin; HIGH at boot may brick startup |
| 13 | Yes |  | Yes | ⚠️ | JTAG by default; otherwise usable |
| 14 | Yes |  | Yes | ⚠️ | JTAG by default; otherwise usable |
| 15 | Yes |  | Yes | ⚠️ | Strapping pin |
| 16 | Yes |  | Yes | ✅ | Good general-purpose GPIO |
| 17 | Yes |  | Yes | ✅ | Good general-purpose GPIO |
| 18 | Yes |  | Yes | ✅ | SPI default SCK |
| 19 | Yes |  | Yes | ✅ | SPI default MISO |
| 21 | Yes |  | Yes | ✅ | Common I2C SDA |
| 22 | Yes |  | Yes | ✅ | Common I2C SCL |
| 23 | Yes |  | Yes | ✅ | SPI default MOSI |
| 25 | Yes |  | Yes | ✅ | DAC capable |
| 26 | Yes |  | Yes | ✅ | DAC capable |
| 27 | Yes |  | Yes | ✅ | Safe |
| 32 | Yes |  | Yes | ✅ | ADC1; excellent analog pin |
| 33 | Yes |  | Yes | ✅ | ADC1; excellent analog pin |
| 34 | Yes |  | ❌ | ⚠️ | Input only |
| 35 | Yes |  | ❌ | ⚠️ | Input only |
| 36 | Yes |  | ❌ | ⚠️ | Input only |
| 39 | Yes |  | ❌ | ⚠️ | Input only |